Technical Specification (HAND PARALYSIS REHABILITATION SYSTEM)

Components Used

Pneumatic Actuation Subsystem:

12V Diaphragm Air Pump
• Type: Miniature diaphragm-type DC air pump
• Operating Voltage: 7 V DC
• Rated Current: 1.5 A – 2.0 A
• Flow Rate: 2.5 – 3.5 L/min (at 12 V)
• Pressure Range: 0 – 2 atm
• Motor Type: DC brushed motor
• Noise Level: < 60 dB
• Use: Air supply for pneumatic actuation

Pneumatic Solenoid Valve (×2)
• Type: Mini 2-position, 3-way solenoid valve
• Operating Voltage: 12 V DC
• Rated Current: 200 – 300 mA
• Port Size: 1/8"
• Pressure Range: 0 – 0.8 MPa
• Response Time: < 15 ms
• Operation Type: Normally closed (NC)
• Material: Brass body, nitrile seals
• Use: Controls air passage for inflation/deflation

Wrist Actuation:

High Torque Servo Motors (×3)
• Model: Surpass Hobby S2500M
• Torque: 25 kg·cm @ 6.8 V
• Speed: 0.15 s/60° @ 6.8 V
• Operating Voltage: 4.8 – 7.4 V DC
• Control Signal: PWM (50 Hz standard)
• Rotation Range: 0° – 180°
• Gear Material: Metal
• Bearing: Dual ball bearing
• Use: Wrist actuation and motion replication

Servo Mounting Brackets and Hinges
• Type: Aluminum/ABS mounting assembly
• Compatibility: Standard 40 × 20 mm servo mounts
• Material: Lightweight aluminum with nylon hinges
• Use: Mechanical joint assembly for servo movement

Electronics and Control:

Microcontroller – ESP32 
• Processor: Dual-core Tensilica Xtensa LX6, up to 240 MHz
• Flash Memory: 4 MB
• SRAM: 520 KB
• Operating Voltage: 3.3 V
• Input Voltage (USB): 5 V
• Wi-Fi Standard: IEEE 802.11 b/g/n (2.4 GHz)
• Bluetooth: v4.2 (LE)
• GPIO Pins: 30 total (ADC/PWM/I2C/SPI/UART capable)
• Logic Level: 3.3 V
• Programming Interface: USB Type-C / UART
• Typical Current Consumption: 80–200 mA (active Wi-Fi)

4-Channel Relay Module (×1)
• Operating Voltage: 5 V DC
• Trigger Signal: High/Low level compatible
• Relay Rating: 10 A @ 250 V AC / 10 A @ 30 V DC
• Isolation: Optocoupler input isolation
• Logic Compatibility: 3.3 V and 5 V systems
• Use: Switching high-current components (pumps, valves, etc.)

DC-DC Buck Converter (×4)
• Input Voltage: 4.5 V – 28 V DC
• Output Voltage: 0.8 V – 20 V adjustable
• Maximum Current: 3 A
• Conversion Efficiency: Up to 92%
• Output Ripple: < 50 mV
• Use: Regulated voltage supply for sensors and servos

Sensors:

IMU Sensor – MPU6050 (×1)
• Accelerometer Range: ±2, ±4, ±8, ±16 g
• Gyroscope Range: ±250, ±500, ±1000, ±2000 °/s
• Interface: I2C digital output
• Operating Voltage: 3 V – 5 V DC
• Sampling Rate: Up to 1 kHz
• Resolution: 16-bit ADC per axis
• Use: Hand motion and orientation detection

User Interface:

Potentiometer Module (×1)
• Type: Rotary potentiometer (10 kΩ)
• Operating Voltage: 3.3 V – 5 V DC
• Output: Analog voltage (0 – Vcc range)
• Mount Type: PCB module with header pins
• Use: Manual control and calibration

16×2 Character LCD (×1)
• Display Type: HD44780-compatible LCD
• Character Format: 16 columns × 2 rows
• Backlight: LED (White on Blue)
• Operating Voltage: 5 V DC
• Interface: Parallel (4-bit / 8-bit)

I²C Interface Module (×1)
• Adapter: I²C serial interface for LCD
• Operating Voltage: 3.3 V – 5 V DC
• Communication: I²C protocol
• Typical Current: < 20 mA
• Use: Simplifies LCD connection with ESP32

Power System:

Battery – Li-Po 3300 mAh 3S 25C (×1)
• Type: Lithium Polymer rechargeable pack
• Nominal Voltage: 11.1 V (3S)
• Capacity: 3300 mAh
• Continuous Discharge: 25C (≈ 82.5 A max)
• Connector Type: XT60
• Protection: Overcharge, short-circuit, reverse polarity
• Use: Power supply for pumps, servos, and ESP32 system

Other Components:

3D Printing and Mounting Structures
• Material: PLA / ABS composite
• Printing Accuracy: ±0.1 mm
• Purpose: Structural enclosure for electronics and pneumatic mounts

Miscellaneous Hardware
• Includes: Wires, connectors, tubing, fittings, and adhesives
• Use: Assembly and integration of all components